BAGNALL-PRICE OUTSIDE ADMISSION SPECIAL INSTRUCTIONS

The common arrangement of the Bagnall-Price gear is with the reverse arm and its lifting arm pointing backwards and the radius rod up for forward.

If the lifting arm is pointing to forward, make the lifting arm negative.

If the radius rod is down for forward then make the eccentric angle  more than 180 degrees.

Remember to recalculate the reverse arm center position if dimensions around the link and the reverse lever have been changed and the auto calculation was not activated.