CAPROTTI     SPECIAL INSTRUCTIONS

SPECIAL CAPROTTI NOTES

Sections of the graphics in graphic motion are moved for viewing

on a two dimensional screen.

There are small errors in the cam-roller contact. For simplicity

several compromises have been taken. The cams have a linear rise

and pointed corners. The roller moves in a horizontal motion not in

an arc. The rollers of the open and close cams have a factor added

to their position to compensate for their offset above and below

the cam shaft center line bringing their tangent point in contact

with the max or min cam radius. All cams have the same linear rise

angle between max and min radius.

The fulcrum pin position is set vertically by the vert length of

the exhaust bellcrank (the vertical length of the steam bellcrank being equal) and horizontally by the average level of the

exhaust cam and the exhaust roller.

The edge view of the cams shows the reversing crank from the side

and the rollers from the top.

The reverse angle is the angle of the rear reverse crank to the

horizontal.

There are two positions of the exhaust cam when the gear is at

center position. One for forward and the other for reverse.

The graphs in this gear do not have the smooth shape of non cam

valve gears. The cams create abrupt changes so interpolation does not work. The graph of port open/close points uses a higher sampling resolution to help smooth the graph.

The percent cutoff data on the main screen shows errors when the step angle is greater than one degree.